IoT Project 2

ESP8266 + DHT11 + LDR + MQ135 + HC-SR04 + OLED + LED Control

MQTT | HiveMQ | Node-RED Dashboard | Live Monitoring

Author: Ramsudarshan Maurya

Company: UniConverge Technologies

GitHub: github.com/Ramsudarshanmaurya

Portfolio: ramsudarshanmaurya.engineer

# 1. Project Overview

This project is an advanced IoT monitoring system using ESP12-E (ESP8266) microcontroller. It monitors Temperature, Humidity, Light, Air Quality, and Distance in real-time. Data is published via MQTT to HiveMQ broker and visualized on Node-RED Dashboard.

Key Features:

Real-time sensor data on Node-RED Dashboard

OLED Display shows all sensor values locally

OLED Alert screen when object detected within 15cm

LED1 (D4) auto-triggers when object detected within 15cm

LED2 (D8) remotely controlled via Node-RED Switch

MQTT Protocol with HiveMQ public broker

# 2. Hardware Used

| Component | Description | Qty |
| --- | --- | --- |
| ESP12-E (ESP8266) | WiFi Microcontroller | 1 |
| DHT11 | Temperature & Humidity Sensor | 1 |
| LDR Module | Light Sensor (Digital Output) | 1 |
| MQ135 | Air Quality / Gas Sensor (Analog) | 1 |
| HC-SR04 | Ultrasonic Distance Sensor | 1 |
| SSD1306 OLED | 128x64 I2C Display | 1 |
| LED 1 | Ultrasonic Trigger LED (<=15cm) | 1 |
| LED 2 | Node-RED Switch Controlled LED | 1 |
| 220 Ohm Resistor | Current Limiting for LEDs | 2 |
| 10K Resistor | Pull-down for D4 LED | 1 |
| 1K Resistor | Voltage Divider ECHO pin | 1 |
| 2K Resistor | Voltage Divider ECHO pin | 1 |
| Jumper Wires | Connections | As needed |
| Breadboard | Circuit Assembly | 1 |

# 3. Pin Connections

| Component | Component Pin | ESP8266 Pin |
| --- | --- | --- |
| DHT11 | VCC | 3.3V |
| DHT11 | GND | GND |
| DHT11 | DATA | D5 (GPIO14) |
| LDR Module | VCC | 3.3V |
| LDR Module | GND | GND |
| LDR Module | DOUT | D3 (GPIO0) |
| MQ135 | VCC | 3.3V |
| MQ135 | GND | GND |
| MQ135 | AOUT | A0 |
| HC-SR04 | VCC | VIN (5V) IMPORTANT |
| HC-SR04 | GND | GND |
| HC-SR04 | TRIG | D6 (GPIO12) |
| HC-SR04 | ECHO | D7 via Voltage Divider |
| OLED SSD1306 | VCC | 3.3V |
| OLED SSD1306 | GND | GND |
| OLED SSD1306 | SDA | D2 (GPIO4) |
| OLED SSD1306 | SCL | D1 (GPIO5) |
| LED1 | Anode (+) | D4 (GPIO2) |
| LED1 | Cathode (-) | 220 Ohm → GND |
| LED2 | Anode (+) | D8 (GPIO15) |
| LED2 | Cathode (-) | 220 Ohm → GND |

## 3.1 HC-SR04 ECHO Voltage Divider

HC-SR04 runs on 5V but ESP8266 GPIO accepts max 3.3V. Voltage divider is required on ECHO pin to protect ESP8266.

HC-SR04 ECHO  →  [1K Ohm]  →  D7 (ESP8266)

|

[2K Ohm]

|

GND

# 4. MQTT Topics

## 4.1 Publish Topics (ESP to Node-RED)

| Topic | Sensor | Data | Unit |
| --- | --- | --- | --- |
| tempNode | DHT11 | Temperature | Celsius |
| humiNode | DHT11 | Humidity | % |
| ldrNode | LDR | Light / Dark | Text |
| mq135Node | MQ135 | Air Quality | 0-1024 |
| distanceNode | HC-SR04 | Distance | cm |
| led1StatusNode | LED1 | Status | ON/OFF |

## 4.2 Subscribe Topics (Node-RED to ESP)

| Topic | Controls | Data |
| --- | --- | --- |
| led2ControlNode | LED2 External | 1/0 or ON/OFF |

# 5. LED Logic

| LED | Pin | Trigger | Condition |
| --- | --- | --- | --- |
| LED1 | D4 (GPIO2) | HC-SR04 Ultrasonic | Auto ON when object <= 15cm |
| LED2 | D8 (GPIO15) | Node-RED Switch | Manual ON/OFF via MQTT |

# 6. OLED Display

## 6.1 Normal Mode

| Line | Display Content |
| --- | --- |
| Line 1 | Temp : 31.5 C |
| Line 2 | Humi : 24.0 % |
| Line 3 | LDR : Light |
| Line 4 | MQ135: 99 |
| Line 5 | Dist : 42.3 cm |

## 6.2 Alert Mode (Object <= 15cm)

| Line | Display Content |
| --- | --- |
| Line 1 | !! ALERT !! (Large Text) |
| Line 2 | Object Detected! |
| Line 3 | Distance: 9.1 cm |
| Line 4 | LED1 → ON |

# 7. Node-RED Dashboard

## 7.1 Page 1 - Sensors

| Widget | Sensor | Type | Range |
| --- | --- | --- | --- |
| Temperature Gauge | DHT11 | Gauge | 0-50 C |
| Humidity Gauge | DHT11 | Gauge | 0-100 % |
| LDR Status | LDR | Text | Light/Dark |
| Air Quality Gauge | MQ135 | Gauge | 0-1024 ppm |
| Distance Gauge | HC-SR04 | Gauge | 0-200 cm |
| LED1 Status | Ultrasonic | Text | ON/OFF |

## 7.2 Page 2 - Charts (Vertical Layout)

| Chart | Sensor | Color |
| --- | --- | --- |
| Temperature | DHT11 | Blue |
| Humidity | DHT11 | Green |
| Light LDR | LDR | Orange |
| Air Quality | MQ135 | Red |
| Distance | HC-SR04 | Purple |

## 7.3 Page 3 - Control

LED2 Toggle Switch controls LED2 via MQTT topic: led2ControlNode

# 8. Arduino Libraries

| Library | Purpose | Install From |
| --- | --- | --- |
| ESP8266WiFi.h | WiFi Connection | ESP8266 Board Package |
| PubSubClient.h | MQTT Protocol | Library Manager |
| DHT.h | DHT11 Sensor | Adafruit DHT Library |
| Wire.h | I2C Communication | Built-in |
| Adafruit_GFX.h | OLED Graphics | Library Manager |
| Adafruit_SSD1306.h | OLED Driver | Library Manager |

# 9. Setup Guide

## 9.1 Arduino IDE

Board: NodeMCU 1.0 (ESP-12E Module)

Baud Rate: 115200

Install ESP8266 Board Package from Board Manager

Install all libraries from Library Manager

## 9.2 Node-RED

Install: node-red-dashboard package

Import: NodeRED_Flow/flow.json

Deploy and open: 127.0.0.1:1880/ui

## 9.3 MQTT Broker

Server: broker.hivemq.com

Port: 1883

No username/password required (public broker)

# 10. About Author

| Field | Detail |
| --- | --- |
| Name | Ramsudarshan Maurya |
| Company | UniConverge Technologies |
| Location | Noida Sector 62 |
| Email | ramsudarshanmaurya@gmail.com |
| LinkedIn | linkedin.com/in/ramsudarshanmaurya |
| Portfolio | ramsudarshanmaurya.engineer |
| GitHub | github.com/Ramsudarshanmaurya |

Made with love by Ramsudarshan Maurya